Autonomous University Robot Assistant

Michael Fritzer, Muhammed Faseeh,

David Haybarger, Kevin Lee

Interface Control Document

REVISION – 1

30 September 2025

Interface Control Document

for

Autonomous University Robot Assistant

Prepared by:

David Haybarger	        09/30/2025

_____________________________

Author	Date

Approved by:

Michael Fritzer	        10/04/2025

_____________________________

Project Leader	Date

Byung-Jun Yoon	        XX/XX/2025

_____________________________

Professor	Date

Fahrettin Ay		        XX/XX/2025

_____________________________

T/A      	Date

Change Record

| Rev. | Date | Originator | Approvals | Description |
| --- | --- | --- | --- | --- |
| 1 | 09/30/2025 | Michael Fritzer, Muhammed Faseeh, David Haybarger, Kevin Lee | Michael Fritzer,Fahrettin Ay,Byung-Jun Yoon | Draft Release |

Table of Contents

Table of Contents	3

List of Tables	4

List of Figures	4

1. Overview ................................................................................................................................. 5

2. References and Definitions .................................................................................................. 5

2.1. References ................................................................................................................ 5

2.2. Definitions ................................................................................................................. 5

3. Physical Interface................................................................................................................... 7

3.1. Weight ....................................................................................................................... 7

3.1.1. Main Processing Unit and Power System .................................................. 7

3.1.2. Robotic System .......................................................................................... 7

3.1.3  UI Components .......................................................................................... 7

3.1.4  3D Printed Shell ......................................................................................... 7

3.2. Dimensions ............................................................................................................... 8

3.2.1. Main Processing Unit and Power System .................................................. 8

3.2.2. Robotic System .......................................................................................... 8

3.2.3  UI Components .......................................................................................... 8

3.2.4  3D Printed Shell ......................................................................................... 8

3.3. Mounting Locations ................................................................................................... 9

4. Thermal Interface ................................................................................................................... 9

5. Electrical Interface ............................................................................................................... 10

5.1. Primary Input Power ............................................................................................... 10

5.1.1. Main Power System.................................................................................. 10

5.1.2. Charging and Protection........................................................................... 10

5.2. User Control Interface ............................................................................................. 10

5.3. Voltage and Current Levels ..................................................................................... 11

5.3.1. Maximum Values ...................................................................................... 11

5.3.2. Stand-by Values ....................................................................................... 11

5.4. Signal Interfaces ..................................................................................................... 12

6. Communications / Device Interface Protocols ................................................................. 12

6.1. Wireless Communications ...................................................................................... 12

6.1.1. Wi-Fi ......................................................................................................... 12

6.1.2. Bluetooth .................................................................................................. 12

6.2. Host Device ............................................................................................................ 12

6.3. Video Interface ........................................................................................................ 12

6.4. Device Peripheral Interface .................................................................................... 12

List of Tables

Table 1: Main Processing Unit and Power System Weight…………………………………………. 7

Table 2: Robotic System Weight……………………………………………………………………….. 7

Table 3: UI Components Weight……………………………………………………………………….. 7

Table 4: 3D Printed Shell Weight………………………………………………………………………. 7

Table 5: Main Processing Unit and Power System Dimensions……………………………………. 8

Table 6: Robotic System Dimensions…………………………………………………………………. 8

Table 7: UI Components Dimensions…………………………………………………………………. 8

Table 8: 3D Printed Shell Dimensions………………………………………………………………… 8

Table 9: Maximum Voltage and Current Levels………………………………………………………11

Table 10: Stand-by Voltage and Current Levels……………………………………………………...11

List of Figures

# Figure 1: Reference of ARUA Deployment Location………………………………………………… 9

Figure 2: Electrical Interface Diagram……………………………………………………………10

# Overview

The following document for the Autonomous University Robot Assistant (ARUA) details the interfaces between its hardware, software, and peripheral subsystems. This document specifies the physical, thermal, electrical, and communication requirements necessary for integration of the robot’s components. It ensures subsystem compatibility and provides a reference for future scaling, maintenance, and troubleshooting.

# References and Definitions

## References

Refer to section 2.2 of the Functional System Requirements document.

## Definitions

CCA			Circuit Card Assembly

mA				Milliamp

mW				Milliwatt

MHz			Megahertz (1,000,000 Hz)

TBD			To Be Determined

BIT				Built-In Test

CCA			Circuit Card Assembly

EMC			Electromagnetic Compatibility

EMI				Electromagnetic Interference

EO/IR	               	Electro-optical Infrared

FDIR		           Failure Detection, Isolation, and Recovery

FOR			Field of Regard

FOV			Field of View

GPS			Global Positioning System

GUI				Graphical User Interface

Hz				Hertz

ICD				Interface Control Document

kHz				Kilohertz (1,000 Hz)

LCD				Liquid Crystal Display

LED				Light-emitting Diode

mA				Milliamp

MHz			Megahertz (1,000,000 Hz)

MTBF		   	Mean Time Between Failure

MTTR			Mean Time To Repair

mW				Milliwatt

PCB			Printed Circuit Board

RMS			Root Mean Square

TBD				To Be Determined

TTL				Transistor-Transistor Logic

USB			Universal Serial Bus

VME			VERSA-Module Europe

TA				Teaching Assistant

AURA			Autonomous University Robot Assistant

AI				Artificial Intelligence

GPT			Generative Pretrained Transformer

LLM				Large Language Model

NLP				Natural Language Processing

RAG			Retrieval Augmented Generation

PSU			Power Supply Unit

SAM			Segment Anything Model

SOP			Standard Operating Procedure

STT				Speech-to-Text

TTS				Text-to-Speech

UI				User Interface

ESP32			Microcontroller responsible for wireless and low level I/O

Jetson Orin Nano		Embedded AI computer responsible for processing and interface

PWM			Pulse Width Modulation

UART			Universal Asynchronous Receiver/Transmitter

GPIO			General Purpose Input/Output pins

ICD				Interface Control Document

MCU			Motor Control Unit

# Physical Interface

## Weight

3.1.1. Main Processing Unit and Power System

| Component | Weight | Number of Items | Total Weight |
| --- | --- | --- | --- |
| Jetson Orin Nano | 222.26g | 1 | 222.26g |
| Battery Power Supply | TBD | 1 | TBD |
| Custom Power PCB | TBD | 1 | TBD |

Table 1: Main Processing Unit and Power System Weight

3.1.2. Robotic System

| Component | Weight | Number of Items | Total Weight |
| --- | --- | --- | --- |
| Servo Motors | 62g | 3 | 186g |
| ESP32 MCU | TBD | 1 | TBD |
| Servo Microcontroller | 9g | 1 | 9g |

Table 2: Robotic System Weight

3.1.3. UI Components

| Component | Weight | Number of Items | Total Weight |
| --- | --- | --- | --- |
| Digital Screen | TBD | 1 | TBD |
| Camera | TBD | 1 | TBD |
| Microphone | TBD | 1 | TBD |

Table 3: UI Components Weight

3.1.4. 3D Printed Shell

| Component | Weight | Number of Items | Total Weight |
| --- | --- | --- | --- |
| Body | TBD | 1 | TBD |
| Arms | TBD | 2 | TBD |

Table 4: 3D Printed Shell Weight

## Dimensions

3.2.1. Dimensions of Main Processing Unit and Power System

| Component | Length | Width | Height |
| --- | --- | --- | --- |
| Jetson Orin Nano | 100mm | 80.5mm | 48mm |
| Battery Power Supply | TBD | TBD | TBD |
| Custom Power PCB | TBD | TBD | TBD |

Table 5: Main Processing Unit and Power System Dimensions

3.2.2. Dimensions of Robotic System

| Component | Length | Width | Height |
| --- | --- | --- | --- |
| Servo Motors | TBD | TBD | TBD |
| ESP32 MCU | TBD | TBD | TBD |
| Servo Microcontroller | 62.5mm | 25.4mm | 3mm |

Table 6: Robotic System Dimensions

3.2.3. Dimensions of UI Components

| Component | Length | Width | Height |
| --- | --- | --- | --- |
| Digital Screen | TBD | TBD | TBD |
| Camera | TBD | TBD | TBD |
| Microphone | TBD | TBD | TBD |

Table 7: UI Components Dimensions

3.2.4. Dimensions of 3D Printed Shell

| Component | Length | Width | Height |
| --- | --- | --- | --- |
| Body | TBD | TBD | TBD |
| Arms | TBD | TBD | TBD |

Table 8: 3D Printed Shell Dimensions

## Mounting Locations

The Autonomous University Robot Assistant (ARUA) will be primarily deployed in university laboratory environments such as electrical engineering teaching labs. The system will be positioned near or on student workbenches where it has access to both power outlets and wireless connectivity for integration with the campus network.

Figure 1: Reference of ARUA Deployment Location

# Thermal Interface

The Jetson Orin Nano will require a heat sink and optional fan to ensure reliable performance during extended computation. While the ESP32 microcontroller and motor driver circuits will also be exposed to similar lab conditions, their thermal output is minimal and does not require dedicated cooling. Because the Jetson Orin Nano is responsible for the majority of AI processing and higher power consumption, it is the primary component at risk of overheating. Proper ventilation slots will be included in the enclosure, and a small cooling fan may be added to maintain stable operating temperatures during long-duration lab sessions.

# Electrical Interface

Figure 2: Electrical Interface Diagram

## Primary Input Power

### 5.1.1 Main Power System

The Autonomous University Robot Assistant (ARUA) will be powered by a 14.6V 4-cell Li-Po battery pack rated at 5000 mAh. This battery will supply sufficient energy for several hours of continuous lab use. The power distribution system uses step-down regulators to provide 5V for servo motors, 14.6V for the Jetson Orin Nano, and 3.3V for the ESP32 and sensor interfaces.

### 5.1.2 Charging and Protection

An inline fuse and polarity protection circuitry are implemented to prevent damage from accidental miswiring. Battery management modules will regulate charging to ensure safe operation during repeated lab use.

## User Control Interface

The user will interact with ARUA through a touchscreen interface powered by the Jetson Orin Nano. A physical emergency stop button is integrated into the chassis, providing immediate cutoff of motor power in case of malfunction. In addition, remote access is available through a web dashboard when the system is connected to Wi-Fi.

## Voltage and Current Levels

### 5.3.1 Maximum Values

| Component | Voltage (V) | Current (A) | Power (W) |
| --- | --- | --- | --- |
| Jetson Orin Nano | 14.6 | 2.37 | 34.6 |
| ESP32 | 3.3 | 500 | 1.76 |
| Servo Motor (per unit) | 7.4 | 0.1 | 0.74W |
| Touchscreen Display | TBD | TBD | TBD |
| Microphone | TBD | TBD | TBD |
| Speaker | TBD | TBD | TBD |

Table 9: Maximum Voltage and Current Levels

The values in this table represent peak load conditions. These are expected only during periods of maximum computation (Jetson), simultaneous servo movement, and display usage.

### 5.3.2 Stand-by Values

| Component | Voltage (V) | Current (A) | Power (W) |
| --- | --- | --- | --- |
| Jetson Orin Nano | 7 | 1 | 7.95 |
| ESP32 | 3.3 | 150 | 0.5 |
| Touchscreen Display | TBD | TBD | TBD |
| Microphone | TBD | TBD | TBD |
| Speaker | TBD | TBD | TBD |

Table 10: Stand-by Voltage and Current Levels

The standby values represent average consumption under idle or low-demand operation, estimated at roughly 60–70% of total usage.

## 5.4 Signal Interfaces

### 5.3.1 PWM Motor Control

Servo motors are driven via PWM signals generated by the ESP32 and routed through motor control expansion boards. Each PWM line is isolated and regulated to prevent signal distortion.

# Communications / Device Interface Protocols

## Wireless Communications

### 6.1.1. Wi-Fi

The ESP32 provides IEEE 802.11 b/g/n Wi-Fi connectivity, enabling communication with the Jetson Orin Nano and external lab networks. This allows ARUA to interface with cloud services, local data servers, and remote terminals for monitoring.

### 6.1.2. Bluetooth

The ESP32 includes Bluetooth 4.2/5.0 support for optional low-power communication with external devices such as smartphones, tablets, or Bluetooth-enabled lab instruments.

## Host Device

The Jetson Orin Nano includes multiple USB 3.0 ports used for connecting peripherals such as a keyboard, mouse, or debugging interface. These ports also support USB-to-serial adapters for direct sensor integration.

## Video Interface

The Jetson Orin Nano provides HDMI 2.0 output to drive the integrated touchscreen display. An external monitor may also be connected for demonstrations or lab presentations.

## Device Peripheral Interface

Communication between the ESP32 and Jetson Orin Nano is handled via UART, using a custom JSON-based protocol. I²C buses are available for ADC expansion modules and future sensor add-ons, while GPIO pins provide interrupt-driven input from user switches and lab safety sensors.

# Audio and Vision Interface

## Overview

The Audio and Vision Interface defines the data, signal, and software connections between the Jetson Orin Nano processing unit, the microphone/speaker array, and the camera subsystem. These interfaces enable real-time speech recognition (STT), text-to-speech (TTS) synthesis, multilingual translation, image-to-text processing, and multimodal fusion for question-answering.

## Speech Interface

### 7.2.1. Input Hardware

Dual digital MEMS microphones connected via USB audio interface to the Jetson Orin Nano.

### 7.2.2. Output Hardware

4 Ω / 3 W speaker-amplifier module powered from the 5 V rail of the Power PCB.

### 7.2.3. Software Stack

OpenAI Whisper STT engine → Local LLM + RAG backend → TTS synthesis (via Coqui TTS or PyTorch Vocoder).

### 7.2.4. Signal Path

Signal Path:

- Analog audio → ADC (USB audio codec) → Jetson audio driver → Whisper STT.

- AI response → TTS engine → DAC → amplifier → speaker output.

### 7.2.5. Data Format

16-bit PCM, 44.1 kHz sampling rate.

### 7.2.6. Latency Requirement

End-to-end (STT→LLM→TTS) response ≤ 1.0 s for typical single-sentence queries.

## Censorship and Translation Layer

### 7.3.1. Censorship

Real-time lexical filter applied post-STT; profanity or restricted terms are masked before forwarding to the LLM.

### 7.3.2. Translation

Integrated multilingual model detects language from STT output and performs automatic translation to English (or maintains native language for echo playback).

### 7.3.3. Interface Dependency

Operates entirely in software on the Jetson CPU / GPU; no additional hardware interface required.

### 7.3.4. Database Logging

Filtered and translated transcripts stored in the local SQLite database with timestamps.

## Vision Interface

### 7.4.1. Input Hardware

8 MP CSI or USB camera connected to Jetson Orin Nano.

### 7.4.2. Processing Stack

Segment Anything Model (SAM 2) + OpenCV + local LLM for image captioning and contextual query interpretation.

### 7.4.3. Multimodal Fusion

STT text and image embedding combined through LLM token alignment to support speech + image questions.

### 7.4.4. Output

Textual or spoken AI response rendered via TTS module and UI display.

### 7.2.5. Frame Format

YUV or RGB 640 × 480 minimum, 30 FPS.

## Communication Interfaces

### 7.5.1. Jetson ↔ ESP32

UART (JSON commands) for voice-triggered robotic actions

### 7.5.2. Jetson ↔ Database

Local SQLite or MySQL API for speech and image logs.

### 7.5.3. Jetson ↔ Tablet/Web UI

WebSocket or REST API for live speech display, translation mode toggle, and audio control.

## Environmental and Electrical Parameters

| Parameter | Value / Range | Notes |
| --- | --- | --- |
| Microphone Operating Voltage | 5 V DC | Draw < 50 mA per channel |
| Speaker Amplifier Voltage | 5 V DC | Peak 1 A draw under speech output |
| Camera Input Voltage | 5 V DC | < 200 mA draw |
| Audio Signal Type | Digital PCM USB | Isolated from main power noise |
| Operating Temperature | 0–45 °C | Lab ambient conditions |