Lake Bryan Thermal Security System  Rylan Bashinski

John Deisher

Kenneth Payne

James Thibodeaux

MIDTERM REPORT

Table of Contents

Concept of Operations………………………………………………………………………………1  Functional System Requirements………………………………………………………………...13  Interface Control Document……………………………………………………………………….31  Schedule and Validation…………………………………………………………………………...41

Lake Bryan Thermal Security System  Rylan Bashinski

John Deisher

Kenneth Payne

James Thibodeaux

CONCEPT OF OPERATIONS

REVISION - 1

4 October 2017

1

CONCEPT OF OPERATIONS

FOR

Lake Bryan Thermal Security System

APPROVED BY:

_____________________________

Project Leader Date

_____________________________

Prof. S. Kalafatis Date

_____________________________

T/A Date

2

Change Record

Rev .

Date Originator Approvals Description

- 9/22/2017 Lake Bryan  Thermal Security

System

1 10/4/2017 Lake Bryan  Thermal Security

System

Jyothsna  Kurra

Draft Release

Revision 1

3

Concept of Operations Revision 1  Lake Bryan Thermal Security System

Table of Contents

List of Figures .................................................................................................................................... 5 1. Executive Summary .................................................................................................................... 6 2. Introduction ................................................................................................................................... 6

2.1 Background ........................................................................................................................... 6 2.2 Overview ................................................................................................................................ 7 2.3 Referenced Documents and Standards ........................................................................... 8

3 Operating Concept ...................................................................................................................... 8 3.1 Scope ..................................................................................................................................... 8 3.2 Operational Description and Constraints ......................................................................... 8 3.3 System Description .............................................................................................................. 9 3.4 Modes of Operations ......................................................................................................... 10 3.5 Users .................................................................................................................................... 10 3.6 Support ................................................................................................................................ 10

4 Scenarios .................................................................................................................................... 11 4.1 Public Access Lake ............................................................................................................ 11 4.2 Near Coast Tracking and Notifications ........................................................................... 11 4.3 Large Area Intruder Detection System ........................................................................... 11

5 Analysis ....................................................................................................................................... 11 5.1 Summary of Proposed Improvements ............................................................................ 11 5.2 Disadvantages and Limitations ........................................................................................ 12 5.3 Alternatives ......................................................................................................................... 12

4

Concept of Operations Revision 1  Lake Bryan Thermal Security System

List of Figures

Figure 1: Lake Bryan Thermal Security System Block Diagram .......................................................... 7 Figure 2: Mesh Network Overview ....................................................................................................... 9

5

Concept of Operations Revision 1  Lake Bryan Thermal Security System

1. Executive Summary

Controlling access to public water areas is important for public safety. Rather than police or  security officers remaining on site for surveillance, our aim is to provide an autonomous  system that will detect and identify trespassing swimmers. The Lake Bryan Thermal  Security System will address this issue with a long-wave infrared camera that will notify  authorities when people are swimming in a targeted area. The LBTSS will provide more  accurate data about human locations in large bodies of water to narrow the area security  officers have to patrol. We also will limit the number of false notifications via rigorous image  processing and thermal imaging over conventional securities systems. This will drastically  limit the manpower and infrastructure needed on site for security.

2. Introduction

This document is an introduction to Lake Bryan Thermal Security System, a system capable  of monitoring public or private bodies of water at night, and informing authorities of human  trespassers. The LBTSS will serve as a deterrent to trespassers as well as a safety  measure to help authorities locate people in the water. The system will also aid the  efficiency of nightly patrols by alerting security or safety personnel to the locations of people  in the water.

2.1 Background

The most common security systems today are video cameras that run continuously. These  cameras store their data on-site or stream to a monitor, and generally, have an attendant  monitoring them in real time. Because these cameras constantly store data, most  businesses opt for low-end systems to save power, memory storage, and manpower.  Everyday society moves to automate remedial tasks, but effective and efficient automated  security systems have taken few strides.

The Lake Bryan Thermal Security System aims to provide accurate, reliable security  surveillance in areas with limited power and internet access. The system will monitor areas  of interest passively, and when motion is detected will activate the camera to determine the  scope of the intrusion. Not continually capturing data from cameras saves on data storage,  as well as allowing cameras to save on power. A single camera change could save up to  $300 per year over conventional security cameras.

On Saturday, August 26, 2017, two students attending Texas A&M were visiting Lake Bryan  with their friends. They decided to go swimming despite hurricane Harvey flood warnings in  the Bryan area. While in the lake, a sudden current pulled the students deeper into the lake.  Their friends left and found help from nearby police officers who rescued the students and  performed cardiopulmonary resuscitation on them until medical personnel arrived on the

6

Concept of Operations Revision 1  Lake Bryan Thermal Security System

scene. The students were taken to a hospital in critical condition. One of the students died  on August 30, 2017, and the other died on September 3, 2017. Tragedies like this could  potentially be avoided if a system like the Lake Bryan Thermal Security System is  implemented on the lake. Authorities could have been notified that people were swimming  in the lake during a hurricane and could have responded to the scene much faster, possibly  preventing any loss of life.

2.2 Overview

Figure 1: Lake Bryan Thermal Security System Block Diagram

Our system will be used to monitor infrared waves to check for human presence in  restricted or dangerous areas. We will detect this information using infrared cameras  mounted with other sensors to determine human versus non-human presence. Using a  network of cameras and sensors that can transmit information between each other, we can  watch large areas of hard to patrol water lines using minimal human interaction. Each  camera node will be solar powered to help limit the need for extra infrastructure to be  installed. A microcontroller will read data from low power sensors to keep the camera from  having to run constantly and drain the battery. Once a sensor detects a possible trespasser,  the camera will take thermal images and relay them using a secure mesh network to the  main control unit for the image to be processed. After an image has been received by the  control unit it will be run through an algorithm to determine if authorities need to be notified  of a trespasser. Using a network connection, the control unit will send out a notification to  the proper authorities so that action can be taken.

7

Concept of Operations Revision 1  Lake Bryan Thermal Security System

2.3 Referenced Documents and Standards

IEEE Wi-Fi communication standards: IEEE 802.11

C95.1-2005 - IEEE Standard for Safety Levels with Respect to Human Exposure to  Radio Frequency Electromagnetic Fields, 3 kHz to 300 GHz

Rechargeable battery http://www.epectec.com/batteries/battery-standards.html  http://www.solarabcs.org/codes-standards/IEEE/index.html

3 Operating Concept

3.1 Scope

The Lake Bryan Thermal Security System (LBTSS) will empower safety and security  personnel to passively monitor a body of water after posted operating hours. When a  trespasser is detected, a thermal camera captures an infrared image of the target area. This  image will then be sent via an RF network to the main control unit, where a microcontroller  will analyze the image and request more images as needed. After this analysis, if a person  is detected, a website will be updated with a notification to alert authorities about  trespassers. For demonstration and testing purposes, a scaled-down system will be built.  While there are many security cameras on the market and even thermal security systems,  the LBTSS is specifically designed to identify “persons in water” or PIWs. Not only will  LBTSS immediately benefit those wishing to secure pools, ponds or lake; there is also the  potential for military applications when scaled up.

3.2 Operational Description and Constraints

The Lake Bryan Thermal Security System is intended to be used by security personnel or  law enforcement agencies to monitor large bodies of water such as ponds and lakes and  locates trespassers. Several camera and sensor nodes will be installed around the border  of a body of water. The main control unit will receive and process images captured from the  cameras and notify the proper authorities. The system will verify that the swimmers are  human, and provide location information.

The resulting constraints from this operational description are as follows:   The master controller must be stored close enough to the lake to communicate with  the system of cameras as well as have access to an internet connection.   The camera nodes must be placed in an area of optimal sunlight to allow solar  charging of their internal batteries.

The camera and sensor nodes should be directed towards the area of interest.   Each camera sensor node must be within the range of the mesh network, and be  able to communicate with the main control unit.

The budget is $500 which limits the quality of parts that can be used for this project.   System nodes must be robust, durable and capable of functioning in most weather  conditions.

8

Concept of Operations Revision 1  Lake Bryan Thermal Security System

3.3 System Description

Solar Power Network: This subsystem will provide power to the camera and sensor  node as well as the transmission network. It will consist of a battery and solar panel  for each camera node. This solar panel will charge the battery during the day and  the battery will provide the necessary power to the system during operating hours.  This subsystem will help to make the overall system self-sustainable and less  dependent on infrastructure since power lines will not need to be run to every  camera. A backup battery will be incorporated to maintain a minimum power supply  in the event of poor solar irradiance.

Camera and Sensor: Our system will be triggered by the motion sensor. The use of  a sensor to trigger an image capture will reduce the activity factor of the camera, and  reduce power consumption. The sensor will have a specified zone to monitor, and  when the sensor detects motion the thermal camera will capture an infrared image of  a specific area. This image is then transferred to a microcontroller inside the mesh  network.

Mesh Network:

Figure 2: Mesh Network Overview

The mesh network will be used for communication between all devices used for  monitoring a specific location. Using a mesh network, the system can be scaled to a  very large size to help with large remote bodies of water. Our system will be using  radio frequency for data transmission since Wi-Fi or other normal data transmission  options are not going to be viable.

9

Concept of Operations Revision 1  Lake Bryan Thermal Security System

Image Processing: Once an image has been received by the main control unit it will  be analyzed to determine if there is a human swimming in the water. This will be  done by analyzing the thermal data provided in the image, compared to the heat  given off by a human being. If the image analyzer determines that a human is  swimming in the body of water it will pass the information to the web notification  application.

Web Notification Application (TBD): When the system has detected a human  trespassing in the water, the web application will upload the thermal image to a  website. The website will provide the user with the time and location of the triggered  node, as well as the infrared image captured. The proper authorities will be able to  use this information to catch trespassers.

3.4 Modes of Operations

The Lake Bryan Thermal Security System will only have one mode of operation which we  call “automatic”. In this mode the user can define the times when the system will function,  otherwise, it will be based on day and night cycles. While operating in automatic the  sensors will detect motion and activate the camera to take an infrared picture. That picture  will be automatically transferred to the main control unit for processing. After the image is  processed and our algorithm detects human presence a notification will be sent to a website  with the location the picture was taken from, the time it was taken, and the actual image.

3.5 Users

Our monitoring system will be marketed to local government law enforcement or security  agencies in charge of large bodies of water that wish to be more effectively secured. This  automated monitoring system will enable these agencies to better schedule patrols through  hard to reach water edges by notifying them of swimmers’ locations.

The Lake Bryan Thermal Security System could also be marketed to agencies like the coast  guard and installed on buoys to make a network of human detectors off the coast. This  would help in search and rescue at night near the shore by providing a trail of images for a  lost boat.

3.6 Support

Support for the Lake Bryan Thermal Security System will be provided in the form of a  detailed user manual providing information on the system installation, maintenance, and  usage. The user manual will also describe how the user can interface will the online website  to view system alerts and the pictures that triggered them.

10

Concept of Operations Revision 1  Lake Bryan Thermal Security System

4 Scenarios

4.1 Public Access Lake

The primary use of the Lake Bryan Thermal Security System will be for public access lakes  that have restricted hours to visitors. If there is an agency responsible for safety at a lake in  hard to patrol areas they can use our system to help schedule patrols or do spot checks  during the restricted hours. One problem with lakes is that there are a large number of  access points, and patrolling the area completely would require extensive employee time  and resources. Using our automated system will help to show exactly where people are  accessing the lake and allow for more active and accurate patrolling.

4.2 Near Coast Tracking and Notifications

Because our system does not require external power or internet for the camera nodes, they  can be installed on buoys to make an imaging network. Groups like the Coast Guard can  use a network like this while searching for lost mariners near the coast. Using images and  time stamps they can make an accurate timeline for where smaller vessels traveled and  help to narrow the search field.

4.3 Large Area Intruder Detection System

The Lake Bryan Thermal Security System can be deployed on land to help keep large areas  under surveillance. Government groups or private owners that need to track if people  trespass on large pieces of land without wanting to constantly patrol the border can deploy  our system instead. Through the use of the Lake Bryan Thermal Security System networks,  the users will be able to track when people reach the thermal cameras and where  trespassing individuals are coming onto the land to deploy security.

5 Analysis

5.1 Summary of Proposed Improvements

The system will be solar powered which means it will be self-sustainable.   The cameras will be activated by motion sensors, preventing the system from  constantly taking pictures, depleting its energy, and wasting image analyzer  computation time.

Each camera will be programmed to have a delay on the image capturing for when  its sensors are constantly detecting movement. This will further help to limit the  energy usage and computational load of the system.

The system provides both an alert and the image that triggered it, allowing users to  better determine the need for security and safety personnel to investigate the body  of water.

The system will run on a predetermined schedule and will not require personal  interaction to maintain upkeep.

11

Concept of Operations Revision 1  Lake Bryan Thermal Security System

5.2 Disadvantages and Limitations

The Lake Bryan Thermal Security System will have some limitations which include:   The system will be optimized to detect adult swimmers and may have difficulty  detecting infants or small children.

Due to budget constraints, the camera range will be limited.

The system mainly surveys along the edges of the water and may not detect people  far from the camera.

The camera nodes should be placed high to decrease the possibility of vandalism or  flood damage.

Camera nodes must be placed in areas with high sunlight to allow the solar panels to  generate energy for the system.

5.3 Alternatives

Some alternatives to the Lake Bryan Thermal Security System are:

Conventional Security System monitored by a user.

No security system just signs deterring people from swimming at night.   Hiring security personnel to make nightly patrols of the body of water.   Power for the system can be provided by running power lines to each individual  camera node.

Transmission could be made through a WIFI system or by hardwiring all of the  camera nodes together to allow communication.

12

Lake Bryan Thermal Security System  Rylan Bashinski

John Deisher

Kenneth Payne

James Thibodeaux

FUNCTIONAL SYSTEM REQUIREMENTS

REVISION – 1

4 October 2017

13

FUNCTIONAL SYSTEM REQUIREMENTS

FOR

Lake Bryan Thermal Security System

APPROVED BY:

_____________________________

Project Leader Date

_____________________________

Prof. S. Kalafatis Date

_____________________________

T/A Date

14

Functional System Requirements  Lake Bryan Thermal Security System

Revision - 1

Change Record

Rev .

Date Originator Approvals Description

- 9/26/2017 Lake Bryan  Thermal

Security System

1 10/4/2017 Lake Bryan  Thermal

Security System

Draft Release

Revision 1

15

Functional System Requirements

Lake Bryan Thermal Security System

Table of Contents

Revision - 1

List of Tables ...................................................................................................................................... 17 List of Figures .................................................................................................................................... 18 1. Introduction ................................................................................................................................. 19

1.1. Purpose and Scope ........................................................................................................... 19 1.2. Responsibility and Change Authority .............................................................................. 20 2. Applicable and Reference Documents .................................................................................. 20 2.1. Applicable Documents ...................................................................................................... 20 2.2. Reference Documents ...................................................................................................... 21 2.3. Order of Precedence ......................................................................................................... 21 3. Requirements ............................................................................................................................. 21 3.1. System Definition ............................................................................................................... 21 3.2. Characteristics .................................................................................................................... 23 3.2.1. Functional / Performance Requirements ................................................................ 23 3.2.2. Physical Characteristics ............................................................................................ 24 3.2.3. Electrical Characteristics ........................................................................................... 25 3.2.4. Environmental Requirements ................................................................................... 27 3.2.5. Failure Propagation .................................................................................................... 27 4. Support Requirements .............................................................................................................. 28 Appendix A Acronyms and Abbreviations ..................................................................................... 29 Appendix B Definition of Terms ...................................................................................................... 30 Appendix C Interface Control Documents ..................................................................................... 30

16

Functional System Requirements

Lake Bryan Thermal Security System

List of Tables

Revision - 1

Table 1: Subsystem Leads ................................................................................................................... 20 Table 2: Applicable Documents ........................................................................................................... 20 Table 3: Reference Documents ............................................................................................................ 21

17

Functional System Requirements

Lake Bryan Thermal Security System

List of Figures

Revision - 1

Figure 1: Project Conceptual Image .................................................................................................... 19 Figure 2: Block Diagram of System .................................................................................................... 22 Figure 3: LBTSS Flowchart ................................................................................................................. 22

18

Functional System Requirements  Lake Bryan Thermal Security System

1. Introduction  1.1. Purpose and Scope

Revision - 1

Controlling access to public water areas after normal operating hours is important for public  safety. Rather than having police or security personnel constantly on site, our aim is to  provide an autonomous system that will detect and identify trespassing swimmers. The  Lake Bryan Thermal Security System will notify authorities of PIW by passively monitoring  the area with infrared thermography (IRT). With our system data can be gathered regarding  where people are swimming in large bodies of water, narrowing the area security personnel  need to maintain. We will filter false-positive notifications out with a fully integrated  processing software system to analyze the thermal images. Our system will limit the  manpower for on-site security, cumbersome infrastructure, and data communication lines.

Figure 1: Project Conceptual Image

19

Functional System Requirements

Lake Bryan Thermal Security System

1.2. Responsibility and Change Authority

Revision - 1

The team leader, John Deisher, will be responsible for verifying all requirements of the  project are met. These requirements can only be changed with the approval of the team  leader and Professor Stavros Kalafatis.

Subsystem Responsibility

Solar Power Network Kenneth Payne

Camera and Sensor James Thibodeaux

Mesh Network John Deisher

Image Processing Rylan Bashinski

Web Notification Application Kenneth Payne

2. Applicable and Reference Documents

2.1. Applicable Documents

The following documents, of the exact issue and revision shown, form a part of this  specification to the extent specified herein:

Document Number Revision/Release Date Document Title

IPC A-610E Revision E – 4/1/2010 Acceptability of Electronic Assemblies  802.11 Feb 9, 2011 Wireless Network Management  ANSI C119.6 2011 American National Standard for Electric  Connectors—Non-Sealed, Multiport Connector

Systems Rated 600 V or Less for Aluminum

and Copper Conductors

PC95.1 2005 Standard for Safety Levels With Respect to  Human Exposure to radio frequency

Electromagnetic Fields

20

Functional System Requirements  Lake Bryan Thermal Security System

2.2. Reference Documents

Revision - 1

The following documents are reference documents utilized in the development of this  specification. These documents do not form a part of this specification and are not  controlled by their reference herein.

Document Number Revision/Release  Date

Document Title

C95.6 October 23, 2002 IEEE Standard for Safety Levels With Respect  to Human Exposure to Electromagnetic Fields

IEC 61427 1999 Secondary cells and batteries for solar  photovoltaic energy systems- General

requirements and methods of test

P1901.2a October 30, 2015 IEEE Approved Draft Standard for Low Frequency (less than 500 kHz) Narrowband

Power Line Communications for Smart Grid

Applications

2.3. Order of Precedence

In the event of a conflict between the text of this specification and an applicable document  cited herein, the text of this specification takes precedence without any exceptions.

All specifications, standards, exhibits, drawings or other documents that are invoked as  “applicable” in this specification are incorporated as cited. All documents that are referred  to within an applicable document are for guidance and information only, apart from ICD’s  that have their applicable documents considered to be incorporated as cited.

3. Requirements

3.1. System Definition

The Lake Bryan Thermal Security System is a sustainable and reliable security system for  after-hour lake monitoring. It allows users to be alerted to the presence of swimmers at  lakes during the night without needing on-site personnel through the use of an online web  application. The Lake Bryan Thermal Security System has five subsystems: Solar Power  Network, Camera and Sensor Module, Mesh Network, Image Processor, and Web  Application.

21

Functional System Requirements  Lake Bryan Thermal Security System

Revision - 1

Figure 2: Block Diagram of System

There will be a solar power system in each camera node that supplies power for the  camera, sensors, microcontroller, and radio frequency (RF) network router. The solar power  will charge a battery during the daytime for nighttime use. The sensors will act as a low power trigger for the camera. When they detect motion the microcontroller will make the  camera capture a thermal image. Once the microcontroller has received the picture from the  camera it will convert it into a bitmap to be sent through the RF mesh network. Each camera  node will have an RF transmitter acting as a router to pass images to the main coordinator  node.

Figure 3: LBTSS Flowchart

When a coordinator receives the image it passes the bitmap to the image processing  microcontroller. The image processor uses the thermal data to determine if a human is  present in the image. If an image passes inspection the microcontroller will upload the  image to a database with information including when and where the image was captured.  Once in the database, a web application will take the image and information and form an  alert on a website that can be monitored by a dispatch officer to inform patrolmen about  where swimmers are at the lake.

22

Functional System Requirements

Lake Bryan Thermal Security System

3.2. Characteristics

3.2.1. Functional / Performance Requirements  3.2.1.1. Standby Wake-Up Miss Rate

Revision - 1

The maximum number of missed trigger incidences in the camera’s field of view and range  shall be less than 5%.

Rationale: The system must consider humans wearing heat blocking clothing, possible  misses from people with large amounts of hair that block heat or other environmental  effects. The MCU providing false positives is a less severe issue than the camera  unreliably triggering.

3.2.1.2. False Positive Rate

The image processing shall have less than a 20% false positive rate for identifying human  trespassers.

Rationale: This tolerance allows a human to make a final determination on any  borderline cases allowing for near misses to still be investigated. By limiting the rate to  20% we do not constantly send data to the dispatcher to review, and only confirmed  positives and borderline cases need to be reviewed.

3.2.1.3. Battery Operating Time

The camera node shall be able to function on battery power for a period of 12 hours  capturing images 10% of the time.

Rationale: The system needs to be able to function during the entire nighttime cycle  should that be what is required by the user. On average we estimate that the camera will  be triggered no more than 10% of the night time (when power consumption will be  highest), thus making the basis for this requirement.

3.2.1.4. Solar Charging Time

The solar charging system shall be able to provide the power to run a maximum of 8 hours  even in worst solar irradiance situations (i.e. cloudy days in January and December). If not,  there must be a backup battery that can act as a supplement.

Rationale: Days are normally longer than 8 hours, but due to the direction of solar rays  and optical obstructions the solar panels will not be charging the entirety of the day cycle.  Due to this, and possible cloud presence, we have limited both the amount of charging  hours and intensity of the solar power.

3.2.1.5. RF Transmission Time

The RF network shall not spend more than 30 seconds per 3000ft to transmit an image  bitmap from a router to the coordinator.

Rationale: Our system is only as useful as the speed at which dispatchers receive alerts.  Because of this, we have a scaling rule for how large the network becomes so that the  image processor can get the images in time to process and send out an alert.

23

Functional System Requirements  Lake Bryan Thermal Security System

3.2.1.6. Time to Alert

Revision - 1

With an uninterrupted Wi-Fi and power source, the entire system shall take no more than  five minutes to provide the user an alert ready for review.

Rationale: See 3.2.1.5 Rational above.

3.2.1.7. Battery Operating Time

The camera node shall be able to function on battery power for a period of 12 hours  capturing images 10% of the time.

Rationale: The system needs to be able to function during the entire night cycle. We  estimate on average the camera will be triggered no more than 10% of the night time  (when power consumption will be highest), thus making the basis for this requirement.

3.2.2. Physical Characteristics

3.2.2.1. Camera Node Area

The Security System camera nodes will operate in a field of view with a radius of 30 feet  and 50 degrees per camera.

Rationale: This area is intended as a demonstration area because of the quality of the  camera our budget can afford. With a higher resolution infrared camera installed each  node would be able to cover much larger areas.

3.2.2.2. System Area

The system can operate over any area so long as an unbroken chain of camera nodes  leads back to the base unit. If transmission times become too extensive, a new network of  cameras would be started with a new main control unit to reduce transmission time.

Rationale: While the modules can function through as many nodes as are in the network  the more hops data has to make increased the time to send out an alert. Because of this,  networks that stretch out too far (as laid out in the requirement 3.2.1.5) will be divided to  allow for faster transmission times.

3.2.2.3. Installation

The installation information for the Lake Bryan Thermal Security System shall be provided  to the customer through a user manual. Camera nodes and the main base unit will come  preloaded with all network information allowing the customer to simply mount the camera  nodes for operation.

24

Functional System Requirements  Lake Bryan Thermal Security System

3.2.2.4. Volume Envelope

Revision - 1

The volume envelope of a single camera node excluding the solar panel shall not exceed  one cubic foot. The volume envelope of the main control unit shall not exceed a height of 4  inches, a width of 8 inches, and a length of 8 inches.

Rationale: The camera nodes should be small enough to be easily mounted around the  perimeter of a body of water. The main control unit will only consist of a microcontroller  and XBee S2 transmitter.

3.2.2.5. Mounting

The mounting information for the Lake Bryan Thermal Security System shall be captured in  the Lake Bryan Thermal Security System ICD.

Rationale: As the LBTSS mounts to a stationary platform, mounting requires specified  ranges and fields of view.

3.2.2.6. Water Resistance

The Lake Bryan Thermal Security System shall be enclosed in a water-resistant case to  protect electrical elements. The camera node should have a viewport that does not interfere  with imaging.

Rationale: This allows the module to be installed outdoors in rainy environments without  risk to expensive electrical components.

3.2.3. Electrical Characteristics

3.2.3.1. Inputs

The presence or absence of any combination of the input signals in accordance with  ICD specifications applied in any sequence shall not damage the Lake Bryan  Thermal Security System, reduce its life expectancy, or cause any malfunction,  either when the unit is powered or when it is not.

No sequence of command shall damage the Lake Bryan Thermal Security System,  reduce its life expectancy, or cause any malfunction.

Rationale: By design, should limit the chance of damage or malfunction by  user/technician error.

3.2.3.1.1 Power Consumption

The maximum peak power for the system shall not exceed 2 watts.

Rationale: This requirement is to ensure continuous operation of the camera nodes  during the night cycle where they rely on stored battery power.

25

Functional System Requirements  Lake Bryan Thermal Security System

3.2.3.1.2 Input Voltage Level

Revision - 1

The input voltage level for the Lake Bryan Thermal Security System shall be 3.3V.

Rationale: All our subsystems are designed to run off of 3.3V included pre-built modules  like the Raspberry Pi 3 and XBee S2.

3.2.3.1.3 External Commands

The Lake Bryan Thermal Security System shall document all external commands in the  appropriate ICD.

Rationale: The ICD will capture all interface details from the low-level electrical to the  high-level packet format.

3.2.3.2. Outputs

3.2.3.2.1 Data Output

The Lake Bryan Thermal Security System shall include a web application for users to view  alerts and images.

Rationale: The LBTSS information will be readily available to the user through an online  application.

3.2.3.2.2 Diagnostic Output

The Lake Bryan Thermal Security System may include a diagnostic interface for error  logging.

Rationale: Provides the ability to manually control things for debugging and a way to  view/download the node map with associated potential targets.

3.2.3.2.3 Raw Video Output

The Lake Bryan Thermal Security System may have the ability to link to an external drive  through a web application for video recording.

Rationale: This would allow the user to view a live thermal stream of the area. However  implementing this feature would require much more power and a much higher  transmission rate.

3.2.3.3. Connectors

The Lake Bryan Thermal Security System shall follow the American National Standard for  Electrical Connectors ANSI C119.6-2011.

Rationale: Conform to connector standard.

26

Functional System Requirements  Lake Bryan Thermal Security System

3.2.3.4. Wiring

Revision - 1

The Lake Bryan Thermal Security System shall follow the guidelines set forth in the National  Electric Code regarding electrical wiring. Article NEC 675 details the guidelines for  electrically driven or controlled irrigation systems.

Rationale: Conform to wiring standard.

3.2.4. Environmental Requirements

The Lake Bryan Thermal Security System shall be designed to withstand and operate in the  environments and laboratory tests specified in the following section.

Rationale: The LBTSS will need to function in an outdoor setting similar to the weather  conditions found in the city of Bryan, Texas.

3.2.4.1. Pressure (Altitude)

The Lake Bryan Thermal Security System shall be able to function properly at altitudes  ranging from sea level to 400 feet above sea level.

3.2.4.2. Thermal

The Lake Bryan Thermal Security System may be able to function properly in an  environment with temperatures ranging from -20℉ to 120℉.

Rationale: The maximum temperature in Bryan, Texas can reach up to 106℉, and the minimum  temperature can reach 24℉. The range of -20℉ to 120℉ allows a buffer to accommodate  variations.

3.2.4.3. Humidity

The Lake Bryan Thermal Security System may be able to function properly in an  environment with relative humidity ranging from 0% to 100%.

3.2.5. Failure Propagation

The Search and Rescue System shall not allow propagation of faults beyond the Search  and Rescue System interface.

3.2.5.1. Failure Detection, Isolation, and Recovery (FDIR)

The Lake Bryan Thermal Security System may have failure detection in the form of a  system check at startup. Every day when the system is scheduled to turn on, the main  control unit can request a picture from each camera node. Throughout operating hours, the  main control unit can periodically request images from individual cameras and compare  these images against those received during startup. If an image is not received after a  certain amount of time then the main control unit may alert the user that the camera node  may be damaged, out of battery, or even stolen. If an image is received then the main  control unit will compare the image to the previously received image and if the patterns  differ too greatly the user can be alerted that the camera node may be obstructed. The  Lake Bryan Thermal Security System may also include energy level detection to alert the  user of low energy camera nodes.

27

Functional System Requirements  Lake Bryan Thermal Security System

3.2.5.1.1 Built-in Test (BIT)

Revision - 1

The Lake Bryan Thermal Security System may have an internal command that will generate  test signals and evaluate the camera node responses and determine if there is a failure.

3.2.5.1.1.1 BIT Critical Fault Detection

The BIT should be able to detect a critical fault on the Lake Bryan Thermal Security System  90 percent of the time.

Rationale: This requirement would allow LBTSS to detect critical faults and informs the  user.

3.2.5.1.1.2 BIT False Alarms

The BIT should have a false alarm rate of less than 10 percent.

Rationale: This requirement will limit the number of false alerts sent to the user to check.

3.2.5.1.1.3 BIT Log

The BIT may save the results of each test to a log that should be stored in the web  application.

Rationale: Allows the user to monitor failures over time and notice failure trends.

3.2.5.1.2 Isolation and Recovery

The Lake Bryan Thermal Security System should provide for fault isolation and recovery by  enabling a full system reset based upon the result of the BIT.

Rationale: In the case of extreme system errors the LBTSS may need to be fully reset  through the web application.

4. Support Requirements

The Lake Bryan Thermal Security System requires an internet connection in order to  interact with the alert page. Users must provide Wi-Fi and power to the main control unit, in  order for it to interact with a web page. One camera node of the system is comprised of (1)  solar panel, (1) battery, (1) single-chip computer, (1) motion sensor, (1) infrared camera,  and (1) mesh network transmitter. The main control unit consists of (1) mesh network  transmitter and (1) single-chip computer.

28

Functional System Requirements

Lake Bryan Thermal Security System

Appendix A Acronyms and Abbreviations  Below is a list of acronyms and abbreviations used in this project.

Revision - 1

BIT Built-In Test  CCA Circuit Card Assembly  EMC Electromagnetic Compatibility  EMI Electromagnetic Interference  EO/IR Electro-optical Infrared  FDIR Failure Detection, Isolation, and Recovery  FOR Field of Regard  FOV Field of View  GPS Global Positioning System  GUI Graphical User Interface  Hz Hertz  ICD Interface Control Document  IEEE Institute of Electrical and Electronics Engineers  IMA Image Analyzer  IR Infrared Radiation  IRT Infrared thermography  kHz Kilohertz (1,000 Hz)  LBTSS Lake Bryan Thermal Security System  LCD Liquid Crystal Display  LED Light-emitting Diode  mA Milliamp  MCU Main Control Unit  MHz Megahertz (1,000,000 Hz)  MTBF Mean Time Between Failure  MTTR Mean Time To Repair  mW Milliwatt  PCB Printed Circuit Board  PIW Person In Water  RF Radio Frequency  RMS Root Mean Square  TBD To Be Determined  TTL Transistor-Transistor Logic  USB Universal Serial Bus  VME VERSA-Module Europe

29

Functional System Requirements

Lake Bryan Thermal Security System

Appendix B Definition of Terms  Appendix C Interface Control Documents

Revision - 1  30

Lake Bryan Thermal Security System Rylan Bashinski

John Deisher

Kenneth Payne

James Thibodeaux

INTERFACE CONTROL DOCUMENT

REVISION – 1

4 October 2017

31

INTERFACE CONTROL DOCUMENT

FOR

Lake Bryan Thermal Security System

APPROVED BY:

_____________________________

Project Leader Date

_____________________________

Prof. Stavros Kalafatis Date

_____________________________

T/A Date

32

Interface Control Document

Lake Bryan Thermal Security System

Change Record

Rev. Date Originator Approvals Description

Revision - 1

- 9/29/2017 Lake Bryan  Thermal

Security System

1 10/4/2017 Lake Bryan  Thermal

Security System

Draft Release

Revision 1

33

Interface Control Document

Lake Bryan Thermal Security System

Table of Contents

Revision - 1

List of Tables ...................................................................................................................... 35 List of Figures ..................................................................................................................... 36 1. Overview ...................................................................................................................... 37 2. References and Definitions .......................................................................................... 37 2.1. References ............................................................................................................ 37

2.2. Definitions ............................................................................................................. 37

3. Physical Interface......................................................................................................... 37 3.1. Weight ................................................................................................................... 37

3.1.1. Main Control Unit............................................................................................ 37 3.1.2. Camera Node ................................................................................................. 37 3.2. Dimensions ........................................................................................................... 38 3.2.1. Dimensions of Mesh Network Subsystem ...................................................... 38 3.2.2. Dimensions of Camera and Sensor Subsystem ............................................. 38 3.2.3. Dimensions of Image Analyzer Subsystem .................................................... 38 3.3. Mounting Locations ............................................................................................... 38

4. Thermal Interface ......................................................................................................... 39 5. Electrical Interface ....................................................................................................... 39 5.1. Primary Input Power .............................................................................................. 39

5.1.1. Camera Node ................................................................................................. 39 5.1.2. Main Control Unit............................................................................................ 39 5.2. Voltage and Current Levels ................................................................................... 39 5.2.1. Maximum Values ............................................................................................ 39 5.2.2. Stand-by Values ............................................................................................. 40 5.3. Signal Interfaces ................................................................................................... 40 5.3.1. XBee Signal Interface ..................................................................................... 40 5.4. User Control Interface ........................................................................................... 40

6. Communications / Device Interface Protocols .............................................................. 40 6.1. Wireless Communications ..................................................................................... 40

6.1.1. Wi-Fi ............................................................................................................... 40 6.1.2. RF .................................................................................................................. 40 6.2. Device Peripheral Interface ................................................................................... 40

34

Revision - 1

Interface Control Document

Lake Bryan Thermal Security System

List of Tables

Table 1: Main Control Weight ............................................................................................. 37 Table 2: Camera Node Weight ........................................................................................... 37 Table 3: Mesh Network Dimensions ................................................................................... 38 Table 4: Camera and Sensor Dimensions .......................................................................... 38 Table 5: Image Analyzer Dimensions ................................................................................. 38 Table 6: Maximum Voltage and Current Levels .................................................................. 40 Table 7: Stand-by Voltage and Current............................................................................... 40

35

Interface Control Document

Lake Bryan Thermal Security System

List of Figures

Revision - 1

Figure 1: Reference Overview of LBTSS Locations ............................................................ 38 Figure 2: Electrical Interface Diagram ................................................................................. 39

36

Interface Control Document  Project Name

1. Overview

Revision -

This document is provided to detail how the camera node and the main control unit will  interface. It will list all possible inputs, outputs, and how the system manages each. First, an  explanation of inputs from the camera node and how that is transferred to the mesh network  will be detailed. A description of how the mesh network communicates to the main control  unit will follow. Finally, the notification system and its interfaces will be laid out in detail.

2. References and Definitions

2.1. References

Refer to section 2.2 of the Functional System Requirements document.

2.2. Definitions

mA Milliamp

mW Milliwatt

g Gram

MHz Megahertz (1,000,000 Hz)

TBD To Be Determined

LBTSS Lake Bryan Thermal Security System

MCU Main Control Unit

3. Physical Interface

3.1. Weight

3.1.1. Main Control Unit

Component Weight Number of Items Total Weight  XBee S2 Module TBD 1 TBD  Raspberry Pi 3 B TBD 1 TBD

5V, 2.5A, Micro USB  Charger

3.1.2. Camera Node

TBD 1 TBD  Table 1: Main Control Weight

Component Weight Number of Items Total Weight  XBee S2 Module TBD 1 TBD

FLiR Dev Kit

KIT-13233 Lepton

0.55g 1 0.55g

Sensor TBD TBD TBD TBD  Microcontroller TBD TBD TBD

37

Interface Control Document

Lake Bryan Thermal Security System

3.2. Dimensions

Dimensions are in inches.

3.2.1. Dimensions of Mesh Network Subsystem

Component Length Width Height  XBee S2 Module .960 1.087 TBD

3.2.2. Dimensions of Camera and Sensor Subsystem

Component Length Width Height

Revision - 1

FLiR Dev Kit

KIT-13233 Lepton

0.335 0.335 0.220

Sensor TBD TBD TBD TBD  Raspberry Pi 0 2.56 1.18 0.669

3.2.3. Dimensions of Image Analyzer Subsystem

Component Length Width Height  Raspberry Pi 3 B 3.370 2.224 0.669  5V, 2.5A, Micro USB TBD TBD TBD  Charger

Table 5: Image Analyzer Dimensions

3.3. Mounting Locations

The LBTSS specifically will be deployed on Lake Bryan in Bryan, Texas. The camera nodes  will need to be positioned near the shore of the lake so that it may have a clear,  unobstructed view while also in a position that receives ample sunlight. The Main Control  Unit will be stationed near the Icehouse utility access point for power and Wi-Fi. For  alternate applications, a similar communication infrastructure access point will be required.

Figure 1: Reference Overview of LBTSS Locations

38

Interface Control Document

Lake Bryan Thermal Security System  4. Thermal Interface

Revision - 1

The Raspberry Pi will use a heat sink for the main control unit. While the other  microcontrollers will also experience similar environmental conditions it is the Raspberry Pi  that will be performing the complex computations. Thus it is the Raspberry Pi that will need  a heat sink to keep it from experiencing decreased efficiency caused by overheating.

5. Electrical Interface

Figure 2: Electrical Interface Diagram

5.1. Primary Input Power

5.1.1. Camera Node

Intended to be an autonomous and self-powered module, the camera node will be powered  by a nine-watt solar panel that feeds at least one five volt Lithium secondary battery. The  capacity will be great enough to hold at minimum two full nights of charge time. There will  also be a backup primary battery that will power the node when total solar irradiance is  insufficient due to weather or time of year.

5.1.2. Main Control Unit

The MCU will be powered by a standard 120 V socket located near the Lake Bryan  Icehouse.

5.2. Voltage and Current Levels

5.2.1. Maximum Values

Component Voltage [V] Current [mA] Power [mW]  XBee S2 3.3 33.0 108.9

FLiR Dev Kit

KIT-13233 Lepton

3.1 45.1 140.0

Raspberry Pi 0 5.0 300 1500  Raspberry Pi 3 B 5.0 600 3000  Sensor TBD TBD TBD

39

Interface Control Document

Lake Bryan Thermal Security System

Table 6: Maximum Voltage and Current Levels

Revision - 1

The values of table 6 are per second to allow for accurate gauging of power consumption at  max power usage. The values represented in this table are maximum power usage which  we estimate to be roughly 10% of the operating time.

5.2.2. Stand-by Values

Component Voltage [V] Current [mA] Power [mW]  XBee S2 3.3 28 92.4

FLiR Dev Kit

KIT-13233 Lepton

3.1 1.3 4.0

Raspberry Pi 3 B TBD TBD TBD

The values of table 7 are per second to allow for accurate gauging of power consumption at  when the module is using its average power consumption. The values represented in this  table are average power usage which we estimate to be roughly 90% of the operating time.

5.3. Signal Interfaces

5.3.1. XBee Signal Interface

The XBee S2 modules will be mounted to a PCB and controlled through using the serial  pins (UART) on our microcontrollers, namely Tx and Rx pins.

5.4. User Control Interface

The user control interface is a web application that communicates the main control unit for  the system. The user’s input will be used to adjust the operating times and access the  diagnostic interface.

6. Communications / Device Interface Protocols  6.1. Wireless Communications

6.1.1. Wi-Fi

The microcontroller has a built-in Wi-Fi module using IEE 802.11 g/b/n standards. This  connection will be used to send alerts to a user web application for review.

6.1.2. RF

The XBee S2 Module used for the Mesh Network will use the protocol: XBee 802.15.4  (Proprietary 802.15.4) for its RF communications between camera nodes and the MCU.

6.2. Device Peripheral Interface

The camera is controlled through a serial port using UART. It will have commands to enter  standby mode as well as to take a picture.

40

Lake Bryan Thermal Security System Rylan Bashinski

John Deisher

Kenneth Payne

James Thibodeaux

SCHEDULE AND VALIDATION

REVISION – 1

4 October 2017

41

Schedule and Validation Revision - 1  Lake Bryan Thermal Security System

Schedule:

Date

Work End date Owner Status  Concept of Operations 9/22/2017 All  Functional System Requirements 9/24/2017 All  Interface Control Document 9/29/2017 All  Project Parts Ordered 9/29/2017 All  Midterm Presentation 10/2/2017 All  John,

James,

Completed

Subsystem Circuit Designs 10/6/2017

Kenneth

Learn HTML 10/2 - 10/6 Kenneth

Pseudocode Subsystem Design 10/6/2017 All

Progress Update 1 10/9/2017 All

Thermal Camera to take in temporary

trigger, capture an image. 10/9/2017 James

Completed circuit drawing for power

supply 10/9/2017 Kenneth

Mesh Network Sends text from

microcontroller nodes to main control

unit microcontroller. 10/9/2017 John

Image Analyzer takes in image as input 10/12/2017 Rylan

Thermal Camera to be able to store

image on SD card under unique names. 10/13/17 James

website skeleton up and running 10/13/17 Kenneth

Camera node microcontroller can take

image as a function parameter and

change it to a bitmap to be sent. 10/13/2017 John

Image Analyzer is able to read the

image as thermal data 10/14/2017 Rylan

Ensure all documentation is up to date 10/15/2017 All

Progress Update 2 10/16/2017 All

Create an object which will store all of

the thermal image data 10/18/2017 Rylan

Testable prototype of power supply for

camera module 10/20/17 Kenneth

42

Schedule and Validation Revision - 1  Lake Bryan Thermal Security System

Thermal Camera to be able to forward

images to mesh network. 10/24/2017 James

Images can be changed back from

bitmaps to usable images after

transmission over the mesh network. 10/24/2017 John

Create code to interpret thermal image

data to determine the different ranges of

heat captured in the image. 10/25/2017 Rylan

Sensor able to register movement in

specific locations and trigger camera. 10/31/2017 James

Create code for receiving image through

the mesh network and passing image

into data structure for holding image. 11/3/2017 John

Website up and testable 11/3/2017 Kenneth

Validation of power supply to meet

specs 11/3/2017 Kenneth

Create code to find heat patterns in the

object storing the thermal data 11/4/2017 Rylan

Progress Update 3 11/6/2017 All

Verification of sensor and camera

communication at ranges up to 30m. 11/9/2017 James

Validation of transmission times and

encoding times for mesh network 11/11/2017 John

Notification System places captured

image with human on the web page

along with the time and position 11/11/2017  Create code to determine if found heat

Rylan,  Kenneth

patterns resembles a person 11/11/2017 Rylan  Progress Update 4 11/13/2017 All  Create code to make final decision if the

image has substantial evidence of

human appearance to alert the user 11/19/2017 Rylan  Verification of full communication

between Sensor, Thermal Camera, and  Mesh Network. 11/21/2017  Notification web page should be able to  store images based on date and time 11/21/2017

James,  John

Rylan,

Kenneth

Work on final presentation 11/22/2017 All

Final validation checks for all individual

subsystems 11/26/2017 All

43

Schedule and Validation Revision - 1  Lake Bryan Thermal Security System

Finish Final Presentation Preparation 11/26/2017 All

Work on Subsystem Presentations 11/27/2017 All

Work on report 11/27/2017 All

Final Presentation 11/28/2017 All

Work on report 11/30/2017 All

Finish Subsystem Presentations 12/3/2017 All

Subsystem Presentations 12/4/2017 All

Finish Final Report 12/5/2017 All

Final Report 12/7/2017 All

Table 1: 403 Schedule

Validation Plan:

Task Specification Result Owner  Peak Power 1.5W Kenneth  Solar Power Supply (max) 1.5W for 12 hours Kenneth

Sensor Read Distance  (min)

10 meters James

Camera Hit Rate (min) 95% James

Total Transmission Time  (max)

Node to Node

Transmission Time (max)  Image Processor False  Positive (max)

Image Processor Positive  Miss (max)

Notification System  response time

5 minutes John

30s per 1000m John

20% Rylan

10% Rylan

<5 minutes Kenneth Payne  44