GPT NLP Lab Assistant Group 3

Michael Fritzer, Muhammed Faseeh,

David Haybarger, Kevin Lee

Functional System Requirements

Functional System Requirements

for the

GPT NLP Lab Assistant Group 3

Team AURA

Prepared by:

Muhammed Faseeh	        10/03/2025

_____________________________

Author	Date

Approved by:

Michael Fritzer	        12/06/2025

_____________________________

Project Leader	Date

Byung-Jun Yoon	        12/06/2025

_____________________________

Professor	Date

### Fahrettin Ay		        12/06/2025

_____________________________

T/A      	Date

Change Record

| Rev. | Date | Originator | Approvals | Description |
| --- | --- | --- | --- | --- |
| 1 | 09/30/2025 | Michael Fritzer, Muhammed Faseeh, David Haybarger, Kevin Lee | Michael Fritzer,Fahrettin Ay,Byung-Jun Yoon | Draft Release |
| 2 | 12/5/2025 | Michael Fritzer, Muhammed Faseeh, David Haybarger, Kevin Lee | Michael Fritzer,Fahrettin Ay,Byung-Jun Yoon | Final Release |

Table of Contents

Table of Contents ............................................................................................................... III

List of Tables ...................................................................................................................... IV

List of Figures ..................................................................................................................... V

1.	Introduction …………………………………………………………………………………...	1

1.1.	Purpose and Scope ………………………………………………………………………	1

1.2.	Responsibility and Change Authority …………………………………………………..	2

2.	Applicable and Reference Documents …………………………………………………...	3

2.1.	Applicable Documents …………………………………………………………………...	3

2.2.	Reference Documents …………………………………………………………………...	4

2.3.	Order of Precedence ……………………………………………………………………..	4

3.	Requirements ………………………………………………………………………………....	5

3.1.	System Definition ………………………………..…………………………………..……	5

3.2.	Characteristics ………………………………..……………………………….……….…	6

3.2.1.	Functional / Performance Requirements ………………………………………....	6

3.2.2.	Physical Characteristics ………………………………………………………….…	7

3.2.3.	Electrical Characteristics …………………………………………………………...	7

3.2.4.	Failure Propagation ………………………………………………………………...	9

4.	Support Requirements …………………………………………………………………….	11

Appendix A Acronyms and Abbreviations ………………………………………………….	13

# List of Tables

TABLE 1: Subsystem Leads ................................................................................................ 2

TABLE 2: Applicable Documents ........................................................................................ 3

TABLE 3: Reference Documents ........................................................................................ 4

# List of Figures

FIGURE 1: Project Conceptual Image …………………………….......….…….…….…......... 1

FIGURE 2: Block Diagram of System ................................................................................. 5

# Introduction

## Purpose and Scope

The Autonomous University Robot Assistant (AURA) is an efficient and interactive system designed to support engineering students in laboratory environments where direct instructor assistance may be limited. The system provides students with real-time academic support by combining artificial intelligence, computer vision, and robotic interaction to assist with circuit troubleshooting, component identification, and guided instruction. Rather than relying solely on professors or teaching assistants, AURA allows students to receive immediate feedback and explanations during hands-on lab activities.

The system gathers data from physical circuits through integrated sensors, probing mechanisms, and a camera system. This information is analyzed by a locally hosted AI model, which delivers actionable feedback and explanations to the student. To ensure accuracy and alignment with official coursework, the system employs Retrieval Augmented Generation (RAG), allowing it to reference curated university lab manuals, SOPs, and technical documents. This ensures that responses are both conversational and grounded in approved instructional material.

The system operates as a bench-top robotic assistant designed for indoor university laboratory use. AURA supports text, voice, and image-based interaction and presents responses through an integrated touchscreen interface and audio output. The system is intended as an instructional support tool to improve student understanding, reduce delays caused by limited instructor availability, and enhance overall learning efficiency in engineering laboratory environments.

Figure 1: Project Conceptual Image

## Responsibility and Change Authority

The team leader, Michael Fritzer, will be responsible for verifying all requirements of the project are met. These requirements can only be changed with the approval of the team leader and Professor Byung-Jun Yoon.

| Subsystem | Responsibility |
| --- | --- |
| Artificial Intelligence | Michael Fritzer |
| Hardware & Power Systems | Muhammed Faseeh |
| User Interface & Web Dashboard | Muhammed Faseeh |
| Robotics | David Haybarger |
| Voice and Image Recognition | Kevin Lee |

Table 1: Subsystem Leads

# Applicable and Reference Documents

## Applicable Documents

The following documents, of the exact issue and revision shown, form a part of this specification to the extent specified herein:

| Document Number | Revision/Release Date | Document Title |
| --- | --- | --- |
| DOD-HDBK-791 | 3/17/1998 | Maintainability Design Techniques Metric |
| MIL-HDBK-217 | Revision F – 2/28/1995 | Reliability Prediction of Electronic Equipment |
| MIL-HDBK-263 | Revision B – 7/31/1994 | Electrostatic Discharge Control Handbook for Protection of Electrical and Electronic Parts, Assemblies and Equipment |
| MIL-HDBK-338 | Revision B – 10/1/1998 | Electronic Reliability Design |
| IPC A-610E | Revision E – 4/1/2010 | Acceptability of Electronic Assemblies |
| MIL-DTL-38999 | Revision L – 5/10/2012 | General Specification for Connectors, Electrical, Circular, Miniature, High Density, Quick Disconnect (Bayonet, Threaded, and Breech Coupling), Environment Resistant, Removable Crimp and Hermetic Solder Contacts |
| MIL-STD-461 | Revision E – 8/20/1999 | Requirements for the Control of Electromagnetic Interface Characteristics of Subsystems and Equipment |

## Table 2: Applicable Documents

## Reference Documents

The following documents are reference documents utilized in the development of this specification.  These documents do not form a part of this specification and are not controlled by their reference herein.

| Document Number | Revision/Release Date | Document Title |
| --- | --- | --- |
| ESP32-TRM | 2020 | Espressif ESP32 Technical Reference Manual |
| MOLICEL-INR18650P26A-V2 | 2020 | Lithium-Ion Rechargeable Battery – INR18650P26A V2 Product Datasheet |
| IEEE Standard 802.11 | 2020 | (Wi-Fi) - Wireless LAN Communication |
| MCP23X17-DS | 2021 | Adafruit MCP23X17 Datasheet |
| LD-2015MG-DS | 2022 | Servo Motor LD-3015MG Datasheet |
| NVIDIA-JetsonNano-DS | 2022 | NVIDIA Jetson Nano Developer Kit Datasheet |
| arXiv:2410.05779 | 2024 | Retrieval-Augmented Generation for Large Language Models: A Survey |
| META-SAM2 | 2024 | Segment Anything Model 2 (SAM 2) — Meta AI |
| GITHUB-RAG-LOCAL-LLM | 2025 | Local LLM with RAG Implementation — A. Scotti |
| Bluepad32-LIB | 2025 | Bluepad32 Library Documentation |
| ALTIUM-PCB-TUT | 2025 | Altium Designer Tutorial – Creating and Configuring PCB Document |

Table 3: Reference Documents

## Order of Precedence

In the event of a conflict between the text of this specification and an applicable document cited herein, the text of this specification takes precedence without any exceptions.

All specifications, standards, exhibits, drawings or other documents that are invoked as “applicable” in this specification are incorporated as cited.  All documents that are referred to within an applicable report are considered to be for guidance and information only, except ICDs that have their relevant documents considered to be incorporated as cited.

# Requirements

## System Definition

The AURA system is an autonomous, bench-top robotic assistant intended for use in university laboratory environments. It integrates artificial intelligence (AI), computer vision, multimodal input/output (I/O), embedded control, and robotic actuation to assist students with laboratory measurements, troubleshooting, and procedural guidance. The system provides instructional support through natural language interaction, physical probing and sensing, and context-aware responses derived from curated academic resources.

Figure 2:  Block Diagram of System

At a high level, the AURA system is composed of the following subsystems (see Figure 2):

- Input and Interaction Subsystem
Accepts student queries through voice, text, and image-based inputs and serves as the primary interface for initiating interactions and uploading lab-related content.

- Vision and Audio Processing Subsystem
Performs speech-to-text (STT), text-to-speech (TTS), and image recognition using integrated cameras and microphones to support conversational interaction and circuit/component recognition.

- AI Assistant Backend Subsystem
Provides the core intelligence of the system using a locally hosted large language model (LLM) with Retrieval Augmented Generation (RAG) for context-aware instructional responses.

- Knowledge Base and Documentation Subsystem
Stores, manages, and serves curated reference material including lab manuals, SOPs, and course documents for use by the AI backend.

- System Control and User Interface Subsystem
Provides a touchscreen-based graphical user interface for direct interaction, file management, and system configuration.

- Peripheral Integration Subsystem
Managed by an ESP-32 microcontroller and supports motors, LEDs, and expressive outputs for robotic interaction and feedback.

- Support and Safety Subsystem
Provides firmware control, system logging, and Built-In Test (BIT) functionality to support fault detection, isolation, and recovery (FDIR).

- Power Systems and Management Subsystem
Supplies regulated power using battery and external power inputs, including kill switch functionality and stable power distribution to the Jetson, ESP32, and all peripherals.

- Web and Portal Access Subsystem
Provides remote access through a secure web-based dashboard that allows students and instructors to review interaction logs, upload instructional documents for RAG usage, and monitor system activity.

## Characteristics

### Functional / Performance Requirements

#### Query Response

The system shall process and provide an initial response to student queries using natural language within 3 seconds for text-based input and within 5 seconds for audio or image-based input under nominal operating conditions.

Rationale:  This response time ensures minimal disruption to laboratory workflows and supports real-time system usability.

#### Knowledge Retrieval

The system shall employ Retrieval Augmented Generation (RAG) to retrieve supporting information from curated course documents, laboratory manuals, and Standard Operating Procedures (SOPs) for at least 90% of student queries under nominal operating conditions.

Rationale:  This ensures that the majority of system responses are grounded in authoritative academic material and aligned with official instructional content.

#### Multimodal Input

The system shall support text-based, voice-based, and image-based input as user interaction modalities.

Rationale:  This increases system accessibility and provides flexible interaction for diverse student needs.

#### Session Logging

The system shall log all user interactions, including student queries and corresponding AI responses, to a local database with associated timestamps.

Rationale: This enables instructors to review student interactions for grading, feedback, and troubleshooting purposes.

### Physical Characteristics

#### Mass

The total mass of the Autonomous University Robot Assistant (AURA) system shall be less than or equal to 7 kilograms.

Rationale:  This ensures the system can be easily placed on and removed from a standard laboratory workbench by a single user without specialized lifting equipment.

#### Volume Envelope

The volume envelope of the Autonomous University Robot Assistant (AURA) system shall be less than or equal to 36 inches in height, 8 inches in width, and 12 inches in length.

Rationale: These dimensional limits ensure compatibility with standard university laboratory workbenches and allow the system to operate without obstructing surrounding equipment or student workspace.

### Electrical Characteristics

#### 3.2.3.1. Inputs

- The presence or absence of any combination of the input signals in accordance with ICD specifications applied in any sequence shall not damage the Autonomous University Robot Assistant System (AURA), reduce its life expectancy, or cause any malfunction, either when the unit is powered or when it is not.

- No sequence of command shall damage the Autonomous University Robot Assistant System, reduce its life expectancy, or cause any malfunction.

Rationale: By design, this limits the chance of damage or malfunction due to user or technician error.

##### Power Consumption

The maximum power consumption of the AURA system shall not exceed 100 watts under full operational load, including Jetson compute, ESP32 peripherals, servo motors, sensors, and user interface operation.

Rationale:  This limit accommodates worst-case operation of all major subsystems (Jetson, servos, and ESP32) while remaining compatible with typical laboratory bench supplies, battery packs, and AC/DC adapters.

##### Input Voltage Level

The system shall operate from a 4-cell lithium-ion battery pack with a nominal voltage of 14.8 VDC and an allowable operating input range of 12.0 VDC to 16.8 VDC at the power entry module (PEM).

For wall-powered operation, the system shall accept input from a regulated AC/DC power adapter providing 19–24 VDC, which shall be converted by an internal DC/DC stage to the required system rails (e.g., Jetson, ESP32, and servos).

Rationale:  This input range matches the expected voltage profile of the selected 4-cell Li-ion pack (MOLICEL INR18650 series) while ensuring compatibility with common bench-top AC/DC adapters and providing sufficient headroom for internal buck regulation to sensitive subsystems.

##### Input Noise and Ripple

The AURA system shall operate correctly when supplied by typical laboratory DC power sources and battery outputs that meet the voltage range specified in 3.2.3.1.2 and exhibit no more than 5% peak-to-peak ripple on the DC output.

Rationale:  This requirement ensures the system remains robust to normal levels of power-supply noise present in bench-top and battery-powered laboratory environments.

##### External Commands

The AURA system shall operate correctly when supplied by either the internal 4-cell lithium-ion battery pack (12.0–16.8 VDC) or a regulated AC/DC laboratory power adapter (19–24 VDC), provided that the input voltage ripple does not exceed 5% peak-to-peak of the nominal input voltage.

Rationale: This requirement ensures robust system operation under normal laboratory battery and bench power conditions without requiring precision noise characterization equipment during validation.

#### Outputs

##### Data Output

The AURA system shall output all relevant system data, including student query information, AI-generated responses, sensor measurements, system status, and operational logs, to the onboard touchscreen user interface and the web/portal interface.

Rationale:  This ensures that users can view real-time responses locally while also enabling remote review of system activity through the web portal.

##### Diagnostic Output

The AURA system shall provide a diagnostic output interface for firmware development, debugging, system monitoring, and data logging through the USB service port and web-based diagnostics dashboard.

Rationale:  This allows developers and maintenance personnel to perform troubleshooting, software updates, and system validation efficiently.

#### Connectors

The AURA system shall utilize keyed, locking electrical connectors rated for the maximum operating voltage and current of each power and signal interface. Power connections shall use high-current locking connectors (e.g., XT60, XT30, or equivalent), while signal and data connections shall use JST, USB, or equivalent laboratory-rated connectors.

All external connectors shall be selected to prevent incorrect mating, reduce the risk of accidental disconnection during normal operation, and ensure safe operation in a university laboratory environment.

Rationale:  These connector types provide sufficient electrical reliability, mechanical retention, and user safety for bench-top laboratory operation without the cost and complexity of military-grade connector systems.

### Failure Propagation

The Autonomous University Robot Assistant (AURA) system shall be designed such that no single internal subsystem failure shall propagate outside of the AURA system boundary or cause damage to external equipment, users, or the laboratory environment.

Rationale:  This ensures that internal faults remain contained within the system and do not create hazards to surrounding lab equipment or users.

#### Failure Detection, Isolation, and Recovery (FDIR)

The AURA system shall implement Failure Detection, Isolation, and Recovery (FDIR) mechanisms to detect internal faults, isolate affected subsystems, and restore safe system operation when possible.

Rationale:  FDIR ensures continued safe operation and allows the system to recover gracefully from internal faults without requiring immediate human intervention.

##### Built In Test (BIT)

The AURA system shall include a Built-In Test (BIT) capability that performs automated health checks on critical subsystems during system startup and periodic operation.

Rationale:  BIT enables early detection of hardware and software failures and supports preventive maintenance.

###### BIT Critical Fault Detection

The BIT shall detect critical system faults with a probability of detection of at least 95 percent.

Rationale:  This detection rate provides a high level of confidence that major system failures will be identified quickly.

###### BIT False Alarms

The BIT shall maintain a false alarm rate of less than 5 percent.

Rationale:  This prevents unnecessary system interruptions and reduces maintenance burden due to incorrect fault indications.

###### BIT Log

The BIT shall log the results of each test, including time of occurrence, subsystem affected, and fault status, to non-volatile system memory for later retrieval by maintenance personnel..

Rationale:  Logged BIT data enables effective troubleshooting and long-term reliability analysis.

##### Isolation and Recovery

Upon detection of a fault, the AURA system shall isolate the affected subsystem and attempt automated recovery through controlled reset, subsystem disablement, or safe-mode operation, depending on fault severity.

If automated recovery is not successful, the system shall enter a safe operational state and notify the user through the touchscreen and web/portal interface.

Rationale: Isolation and controlled recovery prevent cascading failures while ensuring safe behavior during abnormal operating conditions.

# Support Requirements

The Autonomous University Robot Assistant (AURA) system shall be delivered with all necessary hardware, software, and documentation required to ensure proper operation within a university laboratory environment. Support requirements include the following:

4.1. Host Computing Requirements

- A laptop or workstation capable of running Linux or Windows with Python 3.10 or later to support development, debugging, and integration testing with the Jetson Orin Nano.

- Reliable internet access for software package updates and retrieval of external reference materials, including academic papers and updated RAG datasets.

4.2. System Documentation

The AURA system shall be delivered with the following documentation:

- A User Guide detailing startup, shutdown, and normal operating procedures.

- A Maintenance Guide for performing hardware checks (servo calibration, probe reel tensioning, battery replacement) and software updates.

- Source code repositories and hardware schematics provided for continued development, maintenance, and troubleshooting.

- Internet access for software updates and reference material retrieval.

4.3. Technical Support

- During the capstone project lifecycle, the development team shall provide integration support, bug fixes, and system updates.

- After project delivery, the system shall be designed to be self-sufficient, with documentation enabling independent maintenance by future students or faculty.

4.4. Maintenance

- Design documentation and spare components (e.g., servo motors, probe tips, and cabling) shall be provided to support minor repairs.

- The system design shall emphasize modularity, allowing individual subsystems (robotics, AI compute, probe assemblies) to be replaced or upgraded independently.

4.5. Issue Resolution

- Fault detection and logging shall be supported through onboard diagnostics and system logs.

- Troubleshooting procedures shall be documented, including:

- Software recovery (reflashing the Jetson Orin Nano image, resetting ESP32 controllers)

- Hardware resets (probe arm reel disengagement, servo recalibration)

# Appendix A: Acronyms and Abbreviations

AI          	Artificial Intelligence

AURA     	Autonomous University Robot Assistant

BIT         	Built-In Test

CCA         	Circuit Card Assembly

EMC         	Electromagnetic Compatibility

EMI         	Electromagnetic Interference

EO/IR       	Electro-optical Infrared

FDIR        	Failure Detection, Isolation, and Recovery

FOR         	Field of Regard

FOV         	Field of View

GPS         	Global Positioning System

GPT         	Generative Pretrained Transformer

GUI         	Graphical User Interface

Hz         	Hertz

ICD         	Interface Control Document

kHz         	Kilohertz (1,000 Hz)

LCD         	Liquid Crystal Display

LED         	Light-emitting Diode

LLM         	Large Language Model

mA          	Milliamp

MHz         	Megahertz (1,000,000 Hz)

MTBF        	Mean Time Between Failure

MTTR        	Mean Time To Repair

mW          	Milliwatt

NLP         	Natural Language Processing

PCB         	Printed Circuit Board

PSU         	Power Supply Unit

RAG         	Retrieval Augmented Generation

RMS         	Root Mean Square

SAM         	Segment Anything Model

SOP         	Standard Operating Procedure

STT         	Speech-to-Text

TA          	Teaching Assistant

TBD         	To Be Determined

TTL         	Transistor-Transistor Logic

TTS         	Text-to-Speech

UI          	User Interface

USB         	Universal Serial Bus

VME         	VERSA-Module Europe